Satellite Mission Index (ordered by Agency)

## ASI

BISSAT (ASI)	8

IGPM (ASI)	8

COSMO - SkyMed (ASI)	8

Hyperspectral Mission (ASI)	9

BNSC

UK-DMC (BNSC)	9

TopSat (BNSC)	9

TerraSAR-L (BNSC)	9

CAST

HJ-1A (CAST)	10

HJ-1B (CAST)	10

HY-1B (CAST)	10

HJ-1C (CAST)	10

CBERS-2 (CAST / INPE)	11

CBERS-2B (CAST / INPE)	11

CBERS-3 (CAST / INPE)	11

CBERS-4 (CAST / INPE)	12

CNES

Diademe 1&2 (CNES)	12

STARLETTE (CNES)	12

SPOT-2 (CNES)	12

STELLA (CNES)	13

SPOT-4 (CNES)	13

SPOT-5 (CNES)	13

DEMETER (CNES)	14

PARASOL (CNES)	14

PICARD (CNES)	14

Pleiades 1 (CNES)	15

Pleiades 2 (CNES)	15

MEGHA-TROPIQUES (CNES / ISRO)	15

CONAE

SAC-C (CONAE)	15

SAOCOM 1A (CONAE)	16

SAC-F (CONAE)	16

SAOCOM 1B (CONAE)	16

SAC-E/SABIA (CONAE)	16

SAOCOM-2B (2) (CONAE)	17

SAOCOM-2B (1) (CONAE)	17

SAC-D/Aquarius (CONAE / NASA)	17

CSA

RADARSAT-1 (CSA)	17

SCISAT-1 (CSA)	18

RADARSAT-2 (CSA)	18

CSIRO

FedSat (CSIRO / CRCSS)	18

DLR

CHAMP (DLR)	18

BIRD (DLR)	19

TerraSAR-X (DLR)	19

RapidEye (DLR)	19

ESA

ERS-2 (ESA)	19

Envisat (ESA)	20

CRYOSAT (ESA)	20

GOCE (ESA)	20

SMOS (ESA)	21

ADM-Aeolus (ESA)	21

ESA Future Missions (ESA)	21

Swarm (ESA)	22

EUMETSAT

METEOSAT-5 (EUMETSAT)	22

METEOSAT-6 (EUMETSAT)	22

METEOSAT-7 (EUMETSAT)	23

METEOSAT-8 (EUMETSAT)	23

METEOSAT-9 (EUMETSAT)	23

METOP-1 (EUMETSAT)	24

METEOSAT-10 (EUMETSAT)	24

METOP-2 (EUMETSAT)	24

METEOSAT-11 (EUMETSAT)	25

METOP-3 (EUMETSAT)	25

GISTDA

THEOS (GISTDA)	25

INPE

SCD-1 (INPE)	25

SCD-2 (INPE)	26

SSR-1 (INPE)	26

SSR-2 (INPE)	26

ISRO

INSAT-2E (ISRO)	26

IRS-P4 (ISRO)	27

TES (ISRO)	27

KALAPANA (ISRO)	27

INSAT-3A (ISRO)	27

RESOURCESAT-1 (ISRO)	28

CARTOSAT-1 (ISRO)	28

CARTOSAT-2 (ISRO)	28

INSAT-3D (ISRO)	28

RESOURCESAT-2 (ISRO)	29

RISAT-1 (ISRO)	29

OCEANSAT-2 (ISRO)	29

JAXA

ALOS (JAXA)	29

GOSAT (JAXA)	30

GCOM-W (JAXA)	30

GCOM-C (JAXA)	30

GMS-5 (JAXA / JMA)	30

JMA

MTSAT-1R (JMA)	31

MTSAT-2 (JMA)	31

KARI

KOMPSAT-1 (KARI)	31

KOMPSAT-2 (KARI)	31

NASA

LAGEOS-1 (NASA)	32

ERBS (NASA)	32

UARS (NASA)	32

QuikSCAT (NASA)	33

Terra (NASA)	33

ACRIMSAT (NASA)	33

NMP EO-1 (NASA)	34

TIMED (NASA)	34

GRACE (NASA)	34

Aqua (NASA)	35

ICESat (NASA)	35

SORCE (NASA)	35

Aura (NASA)	36

CloudSat (NASA)	36

NMP EO-3 GIFTS (NASA)	36

OCO (NASA)	37

Glory (NASA)	37

DSCOVR (NASA)	37

GPM Core (NASA)	38

GPM Constellation (NASA)	38

HYDROS (NASA)	38

LAGEOS-2 (NASA / ASI)	39

LAGEOS-3 (NASA / ASI)	39

Topex-Poseidon (NASA / CNES)	39

Jason (NASA / CNES)	40

CALIPSO (NASA / CNES)	40

Jason-2 (also known as OSTM) (NASA / CNES)	40

NOAA

NOAA-12 (NOAA)	41

NOAA-14 (NOAA)	41

GOES-9 (NOAA)	41

DMSP F-13 (NOAA)	42

GOES-10 (NOAA)	42

NOAA-15 (NOAA)	42

DMSP F-15 (NOAA)	43

GOES-11 (NOAA)	43

NOAA-16 (NOAA)	43

GOES-12 (NOAA)	44

NOAA-17 (NOAA)	44

DMSP F-16 (NOAA)	44

NOAA-N (NOAA)	45

GOES-N (NOAA)	45

DMSP F-17 (NOAA)	45

NPP (NOAA)	46

GOES-O (NOAA)	46

DMSP F-18 (NOAA)	46

GOES-P (NOAA)	46

NOAA-N' (NOAA)	47

DMSP F-19 (NOAA)	47

NPOESS-1 (NOAA)	47

NPOESS-2 (NOAA)	48

DMSP F-20 (NOAA)	48

GOES-R (NOAA)	48

NPOESS-3 (NOAA)	48

NPOESS-4 (NOAA)	49

NPOESS-5 (NOAA)	49

NPOESS-6 (NOAA)	49

NRSCC

FY-2C (NRSCC)	49

FY-3A (NRSCC)	50

FY-2D (NRSCC)	50

FY-3B (NRSCC)	50

FY-3C (NRSCC)	50

FY-2E (NRSCC)	51

FY-3D (NRSCC)	51

FY-3E (NRSCC)	51

FY-3F (NRSCC)	51

FY-3G (NRSCC)	52

NSAU

SICH-2 (NSAU)	52

SICH-1M (NSAU / ROSKOSMOS)	52

ROSHYDROMET

METEOR-3M N1 (ROSHYDROMET / ROSKOSMOS)	52

Meteor-M No1 (ROSHYDROMET / ROSKOSMOS)	53

METEOR-3M N2 (ROSHYDROMET / ROSKOSMOS)	53

Elektro-L (ROSHYDROMET / ROSKOSMOS)	53

Meteor-M No2 (ROSHYDROMET / ROSKOSMOS)	53

ROSKOSMOS

Vulkan-Kompas-2 (ROSKOSMOS)	54

Monitor-E (ROSKOSMOS)	54

Resurs DK (ROSKOSMOS)	54

Baumanets (ROSKOSMOS)	54

BelKA (ROSKOSMOS)	55

Kanopus-Vulkan (ROSKOSMOS)	55

SNSB

Odin (SNSB)	55

USGS

Landsat-5 (USGS)	55

Landsat-7 (USGS)	56

| # BISSAT (ASI) | Considered | 01 Jan 06 | 01 Jan 08 | Evaluation of bistatic radar cross section of natural and man-made targets, image classification, land surface. Receive-only satellite in formation with main mission COSMO-Skymed | BISSAT | Type: Sun-synchronous Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: |
| --- | --- | --- | --- | --- | --- | --- |
| # IGPM (ASI) | Considered | 01 Jan 06 | 01 Jan 08 | To detect and measure rain and snowfall, to demonstrate the feasibility of high quality measurements of light rain and snow from space | IGPM radiometer, IGPM rain radar | Type: Sun-synchronous Altitude: 510 km Period: Inclination: Repeat cycle: LST: 1430 Longitude (if geo): Asc/desc: TBD URL: |
| # COSMO - SkyMed (ASI) | Planned | 31 Dec 06 | 31 Dec 20 | Environmental monitoring, surveillance and risk management applications, environmental resources management, maritime management, earth topographic mapping, law enforcement, informative / science applications | SAR 2000 | Type: Sun-synchronous Altitude: 619 km Period: 97.86 mins Inclination: Repeat cycle: 16 days LST: 600 Longitude (if geo): Asc/desc: Ascending URL: www.alespazio.it/program/tlr/cosmo/cosmo.htm |
| # Hyperspectral Mission (ASI) | Considered | 01 Jan 09 | 31 Dec 14 | Land surface, agriculture and forestry, regional geology, land use studies, water resources, vegetation studies, coastal studies and soils | CIA, HYC | Type: Sun-synchronous Altitude: 620 km Period: 97 mins Inclination: 91.87 deg Repeat cycle: 16 days LST: 1000 Longitude (if geo): Asc/desc: Descending URL: www.ifac.cnr.it/ot/hyperspectral_workshop_2004.html |
| # UK-DMC (BNSC) | Currently being flown | 27 Sep 03 | 31 Dec 06 | Medium resolution visible imager for support of disaster management | DMC Imager | Type: TBD Altitude: 785 km Period: Inclination: 98.2 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: www.sstl.co.uk/index.php?loc=113 |
| # TopSat (BNSC) | Approved | 01 May 05 | 01 May 06 | Prototype low-cost high-resolution imager | TOPSAT telescope | Type: Sun-synchronous Altitude: 600 km Period: Inclination: 98 deg Repeat cycle: LST: 1030 Longitude (if geo): Asc/desc: TBD URL: www.qinetiq.com/industries/space/spacecraft_technology/case_study_topsat/index.asp |
| # TerraSAR-L (BNSC) | Considered | 01 Jan 06 | 01 Jan 11 | SAR imagery in support of agriculture, forestry etc. | L-SAR | Type: TBD Altitude: 660 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: www.infoterra-global.com/terrasar.html |
| # HJ-1A (CAST) | Planned | 01 Dec 06 | 01 Dec 09 | Disaster and Environment Monitoring and Forecast | CCD (HJ, HY), HSI (HJ-1A) | Type: Sun-synchronous Altitude: 649 km Period: Inclination: 97.9 deg Repeat cycle: 31 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.cast.cn/ |
| # HJ-1B (CAST) | Planned | 01 Dec 06 | 01 Dec 09 | Disaster and Environment Monitoring and Forecast | CCD (HJ, HY), IR (HJ-1B) | Type: Sun-synchronous Altitude: 649 km Period: Inclination: 97.9 deg Repeat cycle: 31 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.cast.cn/ |
| # HY-1B (CAST) | Planned | 01 Dec 06 | 01 Dec 09 | Detecting ocean color and sea surface temperature | COCTS, CZI | Type: Altitude: 798 km Period: Inclination: 98.6 deg Repeat cycle: 7 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.cast.cn/ |
| # HJ-1C (CAST) | Planned | 01 May 07 | 01 May 10 | Disaster and Environment Monitoring and Forecast | S-band SAR | Type: Sun-synchronous Altitude: 499 km Period: Inclination: 97.3 deg Repeat cycle: 31 days LST: 600 Longitude (if geo): Asc/desc: Descending URL: www.cast.cn/ |
| # CBERS-2 (CAST / INPE) | Currently being flown | 22 Oct 03 | 18 Aug 07 | Earth resources, environmental monitoring, land surface (joint with INPE) | CCD (CBERS), DCS (CAST), IR-MSS, WFI | Type: Sun-synchronous Altitude: 778 km Period: 100.26 mins Inclination: 98.5 deg Repeat cycle: 26 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.cast.cn/ & www.cbers.inpe.br/en/programas/cbers1-2.htm |
| # CBERS-2B (CAST / INPE) | Approved | 20 Oct 06 | 20 Oct 08 | Earth resources, environmental monitoring, land surface (joint with INPE) | CCD (CBERS), DCS (CAST), IR-MSS, WFI | Type: Sun-synchronous Altitude: 778 km Period: Inclination: 98.5 deg Repeat cycle: 26 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.cast.cn/ & www.cbers.inpe.br/en/programas/cbers1-2.htm |
| # CBERS-3 (CAST / INPE) | Approved | 20 Oct 08 | 20 Oct 11 | Earth resources, environmental monitoring, land surface (joint with INPE) | DCS (CAST), IRS, MUX, PAN, WFI-2 | Type: Sun-synchronous Altitude: 778 km Period: 100.26 mins Inclination: 98.5 deg Repeat cycle: 26 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.cast.cn/ & www.cbers.inpe.br/en/programas/cbers3-4.htm |
| # CBERS-4 (CAST / INPE) | Approved | 20 Oct 11 | 20 Oct 14 | Earth resources, environmental monitoring, land surface (joint with INPE) | DCS (CAST), IRS, MUX, PAN, WFI-2 | Type: Sun-synchronous Altitude: 778 km Period: 100.26 mins Inclination: 98.5 deg Repeat cycle: 26 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.cast.cn/ & www.cbers.inpe.br/en/programas/cbers3-4.htm |
| # Diademe 1&2 (CNES) | Currently being flown | 15 Feb 67 | 31 Dec 50 | Geodetic measurements using satellite laser ranging | RRA | Type: Inclined, non-sunsynchronous Altitude: 1200 km Period: 108 mins Inclination: 40 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: galileo.crl.go.jp/ilrs/diademe.html |
| # STARLETTE (CNES) | Currently being flown | 06 Feb 75 | 31 Dec 50 | Geodesy/gravity Study of the Earth’s gravitational field and its temporal variations | Laser reflectors | Type: Inclined, non-sunsynchronous Altitude: 812 km Period: 104 mins Inclination: 49.83 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: N/A URL: |
| # SPOT-2 (CNES) | Currently being flown | 22 Jan 90 | 31 Dec 05 | Cartography, land surface, agriculture and forestry, civil planning and mapping, digital terrain models, environmental monitoring | DORIS, HRV | Type: Sun-synchronous Altitude: 832 km Period: 101 mins Inclination: 98.7 deg Repeat cycle: 26 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.spotimage.fr/html/_167_224_229_.php |
| # STELLA (CNES) | Currently being flown | 30 Sep 93 | 31 Dec 50 | Geodesy/gravity Study of the Earth’s gravitational field and its temporal variations | Laser reflectors | Type: Inclined, non-sunsynchronous Altitude: 830 km Period: 101 mins Inclination: 98 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: N/A URL: |
| # SPOT-4 (CNES) | Currently being flown | 24 Mar 98 | 31 Dec 05 | Cartography, land surface, agriculture and forestry, civil planning and mapping, digital terrain models, environmental monitoring | DORIS, HRVIR, VEGETATION | Type: Sun-synchronous Altitude: 832 km Period: 101 mins Inclination: 98.7 deg Repeat cycle: 26 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.spotimage.fr/html/_167_224_229_.php |
| # SPOT-5 (CNES) | Currently being flown | 04 May 02 | 04 May 07 | Cartography, land surface, agriculture and forestry, civil planning and mapping, digital terrain models, environmental monitoring | DORIS-NG, HRG, HRS, VEGETATION | Type: Sun-synchronous Altitude: 832 km Period: 101 mins Inclination: 98.7 deg Repeat cycle: 26 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.spotimage.fr/html/_167_224_229_.php |
| # DEMETER (CNES) | Approved | 29 Jun 04 | 29 Jun 06 | Micro-satellite to study; ionospheric disturbances related to seismic activity, ionospheric disturbances related to human activity, pre and post-seismic effects in the ionosphere, global information on the Earth's electromagnetic environment | IAP, ICE, IDP, IMSC, ISL | Type: Sun-synchronous Altitude: 800 km Period: Inclination: Repeat cycle: LST: 1030 Longitude (if geo): Asc/desc: TBD URL: smsc.cnes.fr/DEMETER/index.htm |
| # PARASOL (CNES) | Approved | 10 Dec 04 | 01 Dec 06 | Micro-satellite with the aim of characterisation of the clouds and aerosols microphysical and radiative properties, needed to understand and model the radiative impact of clouds and aerosols. | POLDER-P | Type: Sun-synchronous Altitude: 700 km Period: 98.8 mins Inclination: Repeat cycle: LST: 1200 Longitude (if geo): Asc/desc: TBD URL: smsc.cnes.fr/PARASOL/index.htm |
| # PICARD (CNES) | Approved | 06 Jan 08 | 01 Aug 10 | Simultaneous measurements of solar diameter, differential rotation, solar constant, and variability. | PREMOS, SODISM, SOVAP | Type: TBD Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: www-projet.cst.cnes.fr:8060/PICARD/index.html |
| # Pleiades 1 (CNES) | Approved | 01 Jul 08 | 01 Jul 13 | Cartography, land use, risk, agriculture and forestry, civil planning and mapping, digital terrain models, defence | Pleiades HR | Type: Sun-synchronous Altitude: 694 km Period: Inclination: Repeat cycle: 26 days LST: 1015 Longitude (if geo): Asc/desc: Descending URL: smsc.cnes.fr/PLEIADES/Fr/index.htm |
| # Pleiades 2 (CNES) | Approved | 31 Dec 09 | 31 Dec 14 | Cartography, land use, risk,agriculture and forestry, civil planning and mapping, digital terrain models, defense | Pleiades HR | Type: Sun-synchronous Altitude: 694 km Period: Inclination: Repeat cycle: LST: 1015 Longitude (if geo): Asc/desc: Descending URL: smsc.cnes.fr/PLEIADES/Fr/index.htm |
| # MEGHA-TROPIQUES (CNES / ISRO) | Approved | 01 Jan 09 | 01 Jan 14 | Study of the inter-tropical zone and its convective systems (water and energy cycles). | MADRAS, SAPHIR, ScaRaB | Type: Sun-synchronous Altitude: 867 km Period: 100 mins Inclination: 20 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: Descending URL: |
| # SAC-C (CONAE) | Currently being flown | 21 Nov 00 | 31 Dec 05 | Earth Observation, studies the structure and dynamics of the Earth’s surface, atmosphere, ionosphere and geomagnetic field. | GOLPE, HRTC, HSTC, ICARE, INES, IST, MMP, MMRS, WTE | Type: Sun-synchronous Altitude: 705 km Period: 98 mins Inclination: 98.2 deg Repeat cycle: 9 days LST: 1015 Longitude (if geo): Asc/desc: Descending URL: www.conae.gov.ar/ |
| # SAOCOM 1A (CONAE) | Approved | 01 Oct 07 | 01 Oct 11 | Earth Observation and Emergency management with an L-band SAR | IR Camera (SAOCOM), SAR (SAOCOM) | Type: Sun-synchronous Altitude: 629 km Period: 96 mins Inclination: 98 deg Repeat cycle: 16 days LST: 600 Longitude (if geo): Asc/desc: Descending URL: www.conae.gov.ar/ |
| # SAC-F (CONAE) | Planned | 21 Nov 07 | 01 Dec 11 | Earth observation studies; emergency management;landscape epidemiology | HRMS, HSMS, HSS, TIS (CONAE) | Type: Sun-synchronous Altitude: 705 km Period: 98 mins Inclination: 98.2 deg Repeat cycle: 9 days LST: 1015 Longitude (if geo): Asc/desc: Descending URL: www.conae.gov.ar/ |
| # SAOCOM 1B (CONAE) | Approved | 01 Oct 08 | 01 Oct 12 | Earth Observation and Emergency management with an L-band SAR | IR Camera (SAOCOM), SAR (SAOCOM) | Type: Sun-synchronous Altitude: 629 km Period: 96 mins Inclination: 98 deg Repeat cycle: 16 days LST: 600 Longitude (if geo): Asc/desc: Descending URL: www.conae.gov.ar/ |
| # SAC-E/SABIA (CONAE) | Planned | 21 Nov 09 | 01 Dec 14 | Food production;environmental monitoring; inner coastal and water quality | MOC | Type: Sun-synchronous Altitude: 705 km Period: 98 mins Inclination: 98.2 deg Repeat cycle: 9 days LST: 1015 Longitude (if geo): Asc/desc: Descending URL: www.conae.gov.ar/ |
| # SAOCOM-2B (2) (CONAE) | Planned | 01 Oct 12 | 01 Oct 16 | Earth Observation and Emergency management with an L-band SAR | IR Camera (SAOCOM), SAR (SAOCOM) | Type: Sun-synchronous Altitude: 629 km Period: 96 mins Inclination: 98 deg Repeat cycle: 16 days LST: 600 Longitude (if geo): Asc/desc: Descending URL: www.conae.gov.ar/ |
| # SAOCOM-2B (1) (CONAE) | Planned | 01 Oct 13 | 01 Oct 17 | Earth Observation and Emergency management with an L-band SAR | IR Camera (SAOCOM), SAR (SAOCOM) | Type: Sun-synchronous Altitude: 629 km Period: 96 mins Inclination: 98 deg Repeat cycle: 16 days LST: 600 Longitude (if geo): Asc/desc: Descending URL: www.conae.gov.ar/ |
| # SAC-D/Aquarius (CONAE / NASA) | Approved | 11 Sep 08 | 11 Sep 11 | Earth observation studies; measurement of ocean salinity; emergency management | Aquarius, HSC, ICARE, MWR (CONAE), NIRST, ROSA, SODAD | Type: Sun-synchronous Altitude: 705 km Period: 98 mins Inclination: 98.2 deg Repeat cycle: 9 days LST: 1015 Longitude (if geo): Asc/desc: Descending URL: www.conae.gov.ar/ |
| # RADARSAT-1 (CSA) | Currently being flown | 04 Nov 95 | 31 Dec 05 | Environmental monitoring, physical oceanography, ice and snow, land surface | RADARSAT DTT, RADARSAT TTC, SAR (RADARSAT) | Type: Sun-synchronous Altitude: 798 km Period: 100.7 mins Inclination: 98.594 deg Repeat cycle: 24 days LST: 1800 Longitude (if geo): Asc/desc: Ascending URL: www.space.gc.ca/csa_sectors/earth_environment/radarsat/default.asp |
| # SCISAT-1 (CSA) | Currently being flown | 12 Aug 03 | 20 Dec 07 | To improve our understanding of the depletion of the ozone layer, particularly over Canada and the Arctic. | ACE-FTS, MAESTRO | Type: Sun-synchronous Altitude: 650 km Period: Inclination: 74 deg Repeat cycle: 15 days LST: Longitude (if geo): Asc/desc: TBD URL: www.space.gc.ca/scisat1 |
| # RADARSAT-2 (CSA) | Approved | 31 Dec 05 | 31 Dec 12 | Environmental monitoring, physical oceanography, ice and snow, land surface | SAR (RADARSAT-2) | Type: Sun-synchronous Altitude: 798 km Period: 100.7 mins Inclination: 98.6 deg Repeat cycle: 24 days LST: 1800 Longitude (if geo): Asc/desc: Ascending URL: www.space.gc.ca/csa_sectors/earth_environment/radarsat2/default.asp |
| # FedSat (CSIRO / CRCSS) | Currently being flown | 14 Dec 02 | 14 Dec 05 | Communications, data relay, near Earth environment, upper atmospheric physics, meteorology | Communications payload (Ka and UHF band), Fluxgate magnetometer, GPS receiver | Type: Sun-synchronous Altitude: 803 km Period: 101 mins Inclination: 98.6 deg Repeat cycle: LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.crcss.csiro.au/ |
| # CHAMP (DLR) | Currently being flown | 15 Jul 00 | 15 Jul 05 | Gravity field, Precise geoid, Magnetic field, Atmospheric physics | CHAMP GPS Sounder, CHAMP gravity package (Accelerometer+GPS), CHAMP magnetometry package (1 Scalar + 2 Vector Magnetometer) | Type: Inclined, non-sunsynchronous Altitude: 470 km Period: Inclination: 87 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: N/A URL: op.gfz-potsdam.de/champ/index_CHAMP.html |
| # BIRD (DLR) | Currently being flown | 22 Oct 01 | 31 Dec 05 | Small satellite mission with technical and scientific objectives (Study of thermal processes on the Earth surface) objectives | HSRS, WAOSS-B | Type: Sun-synchronous Altitude: 572 km Period: Inclination: 97.8 deg Repeat cycle: LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.fire.uni-freiburg.de/iffn/tech/tech_9.htm |
| # TerraSAR-X (DLR) | Approved | 27 Oct 05 | 01 Mar 11 | Cartography, land surface, civil planning and mapping, digital terrain models, environmental monitoring. | TerraSAR-X | Type: Sun-synchronous Altitude: 514 km Period: 94.85 mins Inclination: 97.4 deg Repeat cycle: 11 days LST: 1800 Longitude (if geo): Asc/desc: Ascending URL: www.terrasar.de/ |
| # RapidEye (DLR) | Approved | 01 Jan 07 | 01 Jan 14 | System of 5 satellites for cartography, land surface, digital terrain models, disaster management, environmental monitoring. | MSI | Type: Sun-synchronous Altitude: 600 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: URL: www.rapideye.de/ |
| # ERS-2 (ESA) | Currently being flown | 21 Apr 95 | 31 Dec 05 | Earth resources plus physical oceanography, ice and snow, land surface, meteorology, geodesy/gravity, environmental monitoring, atmospheric chemistry | AMI/SAR/Image, AMI/SAR/wave, AMI/scatterometer, ATSR/M, ATSR-2, ERS Comms, GOME, MWR (BNSC), RA | Type: Sun-synchronous Altitude: 782 km Period: 100.5 mins Inclination: 98.52 deg Repeat cycle: 35 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.esa.int/export/esaEO/SEMGWH2VQUD_index_0_m.html |
| # Envisat (ESA) | Currently being flown | 01 Mar 02 | 01 Mar 07 | Physical oceanography, land surface, ice and snow, atmospheric chemistry, atmospheric dynamics/water and energy cycles | AATSR, ASAR, ASAR (image mode), ASAR (wave mode), DORIS-NG, ENVISAT Comms, GOMOS, MERIS, MIPAS, MWR (BNSC), RA-2, SCIAMACHY | Type: Sun-synchronous Altitude: 782 km Period: 100.5 mins Inclination: 98.52 deg Repeat cycle: 35 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: envisat.esa.int/ |
| # CRYOSAT (ESA) | Approved | 31 Mar 05 | 31 Mar 08 | A radar altimetry mission to determine variations in the thickness of the Earth’s continental ice sheets and marine ice cover. Primary objective is to test the prediction of thinning arctic ice due to global warming | DORIS-NG, Laser reflectors (ESA), SIRAL | Type: Inclined, non-sunsynchronous Altitude: 717 km Period: Inclination: 92 deg Repeat cycle: 369 days LST: Longitude (if geo): Asc/desc: TBD URL: www.esa.int/export/esaLP/cryosat.html |
| # GOCE (ESA) | Approved | 15 Feb 06 | 15 Feb 08 | Research in steady-state ocean circulation, physics of Earth's interior and levelling systems (based on GPS). Will also provide unique data set required to formulate global and regional models of the Earth's gravity field and geoid. | EGG, GPS (ESA), Laser reflectors (ESA) | Type: Sun-synchronous Altitude: 250 km Period: Inclination: 96.5 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: www.esa.int/export/esaLP/goce.html |
| # SMOS (ESA) | Approved | 15 Feb 07 | 15 Feb 10 | Overall objectives are to provide global observations of two crucial variables for modelling the weather and climate, Soil Moisture and Ocean Salinity. It will also monitor the vegetation water content, snow cover and ice structure. | MIRAS (SMOS) | Type: Sun-synchronous Altitude: 756 km Period: Inclination: 98.45 deg Repeat cycle: 165 days LST: 600 Longitude (if geo): Asc/desc: Ascending URL: www.esa.int/export/esaLP/smos.html |
| # ADM-Aeolus (ESA) | Approved | 15 Oct 07 | 15 Oct 10 | Will provide wind profile measurements for global 3-D wind field products used for study of atmospheric dynamics, including global transport of energy, water, aerosols, and chemicals | ALADIN | Type: Sun-synchronous Altitude: 408 km Period: Inclination: 96.99 deg Repeat cycle: 7 days LST: Longitude (if geo): Asc/desc: TBD URL: www.esa.int/export/esaLP/aeolus.html |
| # ESA Future Missions (ESA) | Considered | 01 Jan 08 | 01 Jan 18 | Physical Oceanogreaphy, land surface, ice and snow, atmospheric dynamics/water and energy cycles | ATLID, CPR, MASTER, SPECTRA | Type: TBD Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: www.esa.int/export/esaLP/index.html |
| # Swarm (ESA) | Approved | 01 Jan 09 | 01 Jan 13 | To provide the best ever survey of the geomagnetic field and its temporal evolution, and gain new insights into improving our knowledge of the Earth’s interior and climate. | ACC, ASM, EFI, GPS (ESA), STR, VFM | Type: Inclined, non-sunsynchronous Altitude: 450 km Period: Inclination: 87.4 deg Repeat cycle: LST: 600 Longitude (if geo): Asc/desc: URL: www.esa.int/export/esaLP/swarm.html |
| # METEOSAT-5 (EUMETSAT) | Currently being flown | 02 Mar 91 | 31 Dec 05 | Meteorology, climatology, Atmospheric dynamics/water and energy cycles. Meteosat 1-7 are first generation. Meteosat 8-11 are second generation and known as MSG in the development phase | METEOSAT Comms, MVIRI | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -63 Asc/desc: N/A URL: umetsat.de/en/dps/news/spacecraft.html |
| # METEOSAT-6 (EUMETSAT) | Currently being flown | 20 Nov 93 | 01 Jun 08 | Meteorology, climatology, Atmospheric dynamics/water and energy cycles. Meteosat 1-7 are first generation. Meteosat 8-11 are second generation and known as MSG in the development phase | METEOSAT Comms, MVIRI | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 9 Asc/desc: N/A URL: www.eumetsat.de/en/dps/news/spacecraft.html |
| # METEOSAT-7 (EUMETSAT) | Currently being flown | 03 Sep 97 | 01 Jun 09 | Meteorology, climatology, Atmospheric dynamics/water and energy cycles. Meteosat 1-7 are first generation. Meteosat 8-11 are second generation and known as MSG in the development phase | METEOSAT Comms, MVIRI | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 0 Asc/desc: N/A URL: www.eumetsat.de/en/mtp/index.html |
| # METEOSAT-8 (EUMETSAT) | Currently being flown | 13 Aug 02 | 13 Aug 09 | Meteorology, climatology, Atmospheric dynamics/water and energy cycles. Meteosat 1-7 are first generation. Meteosat 8-11 are second generation and known as MSG in the development phase | GERB, MSG Comms, SEVIRI | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 0 Asc/desc: N/A URL: www.eumetsat.de/en/area4/topic1.html |
| # METEOSAT-9 (EUMETSAT) | Approved | 01 Jun 05 | 01 Jun 12 | Meteorology, climatology, Atmospheric dynamics/water and energy cycles. Meteosat 1-7 are first generation. Meteosat 8-11 are second generation and known as MSG in the development phase | GERB, MSG Comms, SEVIRI | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 0 Asc/desc: N/A URL: www.eumetsat.de/en/area4/topic1.html |
| # METOP-1 (EUMETSAT) | Approved | 01 Dec 05 | 01 Dec 10 | Meteorology, climatology | AMSU-A, ARGOS, ASCAT, AVHRR/3, GOME-2, GRAS, HIRS/4, IASI, MCP, MHS, S&R (NOAA) | Type: Sun-synchronous Altitude: 840 km Period: 101.7 mins Inclination: 98.8 deg Repeat cycle: 5 days LST: 930 Longitude (if geo): Asc/desc: Descending URL: www.eumetsat.de/en/area4/topic2.html |
| # METEOSAT-10 (EUMETSAT) | Approved | 01 Jun 08 | 01 Jun 15 | Meteorology, climatology, Atmospheric dynamics/water and energy cycles. Meteosat 1-7 are first generation. Meteosat 8-11 are second generation and known as MSG in the development phase | GERB, MSG Comms, SEVIRI | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 0 Asc/desc: N/A URL: www.eumetsat.de/en/area4/topic1.html |
| # METOP-2 (EUMETSAT) | Approved | 31 Dec 09 | 31 Dec 14 | Meteorology, climatology | AMSU-A, ARGOS, ASCAT, AVHRR/3, GOME-2, GRAS, HIRS/4, IASI, MCP, MHS, S&R (NOAA), SEM (POES) | Type: Sun-synchronous Altitude: Period: Inclination: Repeat cycle: LST: 930 Longitude (if geo): Asc/desc: N/A URL: www.eumetsat.de/en/area4/topic2.html |
| # METEOSAT-11 (EUMETSAT) | Approved | 01 Dec 11 | 01 Dec 18 | Meteorology, climatology, Atmospheric dynamics/water and energy cycles. Meteosat 1-7 are first generation. Meteosat 8-11 are second generation and known as MSG in the development phase | GERB, MSG Comms, SEVIRI | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 0 Asc/desc: N/A URL: www.eumetsat.de/en/area4/topic1.html |
| # METOP-3 (EUMETSAT) | Approved | 01 Jun 14 | 01 Jun 19 | Meteorology, climatology | AMSU-A, ARGOS, ASCAT, AVHRR/3, GOME-2, GRAS, HIRS/4, IASI, MCP, MHS | Type: Sun-synchronous Altitude: 840 km Period: 101.7 mins Inclination: 98.8 deg Repeat cycle: 5 days LST: 930 Longitude (if geo): Asc/desc: Descending URL: www.eumetsat.de/en/area4/topic2.html |
| # THEOS (GISTDA) | Approved | 01 Jul 07 | 01 Jul 12 | Earth Resources, Land surface and Disaster Monitoring, Civil Planning | MS (GISTDA), PAN (GISTDA) | Type: Sun-synchronous Altitude: 822 km Period: 101 mins Inclination: 98.7 deg Repeat cycle: 35 days LST: 1000 Longitude (if geo): Asc/desc: Descending URL: www.gistda.or.th |
| # SCD-1 (INPE) | Currently being flown | 09 Feb 93 | 01 Dec 05 | Data collection and communication | DCP (SCD) | Type: Inclined, non-sunsynchronous Altitude: 750 km Period: 100 mins Inclination: 25 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: www.inpe.br/programas/mecb/default.htm |
| # SCD-2 (INPE) | Currently being flown | 22 Oct 98 | 01 Dec 05 | Data collection and communication | DCP (SCD) | Type: Inclined, non-sunsynchronous Altitude: 750 km Period: 100 mins Inclination: 25 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: www.inpe.br/programas/mecb/default.htm |
| # SSR-1 (INPE) | Approved | 01 Dec 07 | 01 Dec 11 | Earth resources, environmental monitoring, land surface | OBA | Type: Inclined, non-sunsynchronous Altitude: 905 km Period: 103.2 mins Inclination: 0 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: N/A URL: www.inpe.br/programas/mecb/default.htm |
| # SSR-2 (INPE) | Planned | 01 Dec 12 | 01 Dec 15 | Earth resources, environmental monitoring, land surface | L-band SAR, OBA | Type: Inclined, non-sunsynchronous Altitude: 905 km Period: 103 mins Inclination: 0 deg Repeat cycle: 16 days LST: Longitude (if geo): Asc/desc: TBD URL: www.inpe.br/programas/mecb/default.htm |
| # INSAT-2E (ISRO) | Currently being flown | 03 Apr 99 | 04 Mar 11 | Meteorology, data collection and communication, search and rescue | CCD camera, VHRR | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -83 Asc/desc: TBD URL: www.isro.org/ |
| # IRS-P4 (ISRO) | Currently being flown | 26 May 99 | 31 Dec 06 | Ocean biology, physical oceanography | MSMR, OCM, WiFS | Type: Sun-synchronous Altitude: 720 km Period: 99.31 mins Inclination: 98.28 deg Repeat cycle: 2 days LST: 1215 Longitude (if geo): Asc/desc: Descending URL: www.isro.org/ |
| # TES (ISRO) | Currently being flown | 22 Oct 01 | 31 Dec 05 | For demonstrating many satellite technologies for future Cartosat satellites |  | Type: Sun-synchronous Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: Descending URL: www.isro.org/ |
| # KALAPANA (ISRO) | Currently being flown | 09 Dec 02 | 09 Dec 12 | Meteorological applications | VHRR | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -83 Asc/desc: TBD URL: www.isro.org/ |
| # INSAT-3A (ISRO) | Currently being flown | 10 Apr 03 | 10 Apr 13 | Meteorology, data collection and communication, search and rescue | CCD camera, VHRR | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -94 Asc/desc: TBD URL: www.isro.org/ |
| # RESOURCESAT-1 (ISRO) | Currently being flown | 01 Oct 03 | 01 Oct 08 | Natural Resources Management; Agricultural applications; Forestry | AWiFS, LISS-III, LISS-IV | Type: Sun-synchronous Altitude: 817 km Period: 102 mins Inclination: 98.72 deg Repeat cycle: 26 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: www.isro.org/ |
| # CARTOSAT-1 (ISRO) | Approved | 15 Mar 05 | 01 Jul 09 | High precision large-scale cartographic mapping of 1:10000 scale and thematic applications (with merged XS data) at 1:4000 scales | PAN (Cartosat-1) | Type: Sun-synchronous Altitude: 630 km Period: 97.178 mins Inclination: 97.87 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: Descending URL: www.isro.org/ |
| # CARTOSAT-2 (ISRO) | Approved | 31 Dec 05 | 01 Jan 10 | High precision large-scale cartographic mapping of 1:10000 scale and thematic applications (with merged XS data) at 1:4000 scales | HR-PAN | Type: Sun-synchronous Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: Descending URL: www.isro.org/ |
| # INSAT-3D (ISRO) | Approved | 01 Jul 06 | 01 Jul 15 | Meteorology, data collection and communication, search and rescue | Imager (INSAT), Sounder (INSAT) | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -83 Asc/desc: TBD URL: www.isro.org/ |
| # RESOURCESAT-2 (ISRO) | Planned | 01 Jul 06 | 01 Jul 11 | Natural Resources Management; Agricultural applications; Forestry | AWiFS, LISS-III, LISS-IV | Type: Sun-synchronous Altitude: 817 km Period: 102 mins Inclination: 98.72 deg Repeat cycle: 26 days LST: Longitude (if geo): Asc/desc: Descending URL: www.isro.org/ |
| # RISAT-1 (ISRO) | Approved | 01 Jul 06 | 01 Jul 11 | Land surface, agriculture and forestry, regional geology, land use studies, water resources, vegetation studies, coastal studies and soils - Specially during cloud season | SAR (RISAT) | Type: Sun-synchronous Altitude: 586 km Period: 96.5 mins Inclination: 12 deg Repeat cycle: 600 days LST: Longitude (if geo): Asc/desc: Descending URL: www.isro.org/ |
| # OCEANSAT-2 (ISRO) | Planned | 01 Jan 07 | 01 Jan 11 | Ocean and atmosphere applications | OCM, Scatterometer (ISRO) | Type: Sun-synchronous Altitude: 720 km Period: 99.31 mins Inclination: 98.28 deg Repeat cycle: 2 days LST: 1200 Longitude (if geo): Asc/desc: Descending URL: www.isro.org/ |
| # ALOS (JAXA) | Approved | 01 Sep 05 | 01 Sep 10 | Cartography, digital terrain models, environmental monitoring, disaster monitoring, civil planning, agriculture and forestry, Earth resources, land surface | AVNIR-2, PALSAR, PRISM | Type: Sun-synchronous Altitude: 692 km Period: 98.7 mins Inclination: 98.16 deg Repeat cycle: 3 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: alos.jaxa.jp/index-e.html |
| # GOSAT (JAXA) | Planned | 01 Feb 08 | 01 Feb 13 | Observation of Greenhouse gases | Cloud Sensor, GHG Sensor | Type: Sun-synchronous Altitude: 666 km Period: 98 mins Inclination: 98 deg Repeat cycle: 3 days LST: Longitude (if geo): Asc/desc: TBD URL: www.jaxa.jp/missions/projects/sat/eos/gosat/index_e.html |
| # GCOM-W (JAXA) | Considered | 01 Jan 09 | 01 Jan 14 | Sea surface observation | AMSR follow-on, Scatterometer (JAXA) | Type: Sun-synchronous Altitude: 800 km Period: Inclination: Repeat cycle: LST: 1330 Longitude (if geo): Asc/desc: Descending URL: |
| # GCOM-C (JAXA) | Considered | 01 Jan 10 | 01 Jan 15 | Atmospheric and terrestrial observation | GLI follow-on | Type: Sun-synchronous Altitude: 800 km Period: Inclination: Repeat cycle: LST: 1330 Longitude (if geo): Asc/desc: Descending URL: |
| # GMS-5 (JAXA / JMA) | Currently being flown | 18 Mar 95 | 01 Jun 05 | Meteorology | DCS (JAXA), GMS Comms, VISSR (GMS-5) | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -140 Asc/desc: N/A URL: spaceinfo.jaxa.jp/db/kaihatu/eisei/eisei_e/gms_5_e.html |
| # MTSAT-1R (JMA) | Approved | 01 Feb 05 | 31 Dec 10 | Meteorology, aeronautical applications | IMAGER/MTSAT-1R, MTSAT Comms | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -140 Asc/desc: N/A URL: |
| # MTSAT-2 (JMA) | Approved | 01 Feb 06 | 31 Dec 15 | Meteorology, aeronautical applications | IMAGER/MTSAT-2, MTSAT Comms | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -140 Asc/desc: N/A URL: |
| # KOMPSAT-1 (KARI) | Currently being flown | 21 Dec 99 | 21 Dec 05 | Cartography, land use and planning, disaster monitoring, Global marine resource and environmental monitoring, ocean contamination and chlorophyll detection | EOC, OSMI | Type: Sun-synchronous Altitude: 685 km Period: 98.5 mins Inclination: 28 deg Repeat cycle: 1050 days LST: Longitude (if geo): Asc/desc: Ascending URL: kompsat.kari.re.kr/english/index.asp |
| # KOMPSAT-2 (KARI) | Approved | 01 Oct 05 | 01 Oct 08 | Cartography, land use and planning, disaster monitoring | MSC | Type: Sun-synchronous Altitude: 685 km Period: 98.5 mins Inclination: 28 deg Repeat cycle: 1050 days LST: Longitude (if geo): Asc/desc: Ascending URL: kompsat.kari.re.kr/english/index.asp |
| # LAGEOS-1 (NASA) | Currently being flown | 04 May 76 | 04 May 16 | Geodesy, crustal motion and gravity field measurements by laser ranging | LRA (LAGEOS) | Type: Inclined, non-sunsynchronous Altitude: 6000 km Period: 225 mins Inclination: 110 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: N/A URL: ilrs.gsfc.nasa.gov/satellite_missions/list_of_satellites/lagoes.html |
| # ERBS (NASA) | Currently being flown | 05 Oct 84 | 30 Apr 05 | Earth radiation budget measurements. | ERBE, SAGE II | Type: Inclined, non-sunsynchronous Altitude: 585 km Period: 96.3 mins Inclination: 57 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: N/A URL: asd-www.larc.nasa.gov/erbe/erbs.html |
| # UARS (NASA) | Currently being flown | 15 Sep 91 | 30 Apr 05 | Atmospheric chemistry (middle to upper atmosphere), atmospheric dynamics/water and energy cycles. HALOE, HRDI, MLS, PEM instruments still functioning. End date TBD. | ACRIM II, CLAES, HALOE, HRDI, ISAMS, PEM, SOLSTICE, SUSIM (UARS), WINDII | Type: Inclined, non-sunsynchronous Altitude: 585 km Period: 95.9 mins Inclination: 57 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: N/A URL: umpgal.gsfc.nasa.gov/uars-science.html |
| # QuikSCAT (NASA) | Currently being flown | 19 Jun 99 | 01 Oct 07 | Acquires accurate, high-resolution, global measurements of sea-surface wind vectors in 1 to 2 day repeat cycles for studies of tropospheric dynamics and air-sea interaction processes, including air-sea momentum transfer. End of life date TBD. | SeaWinds | Type: Sun-synchronous Altitude: 803 km Period: 101 mins Inclination: 98.6 deg Repeat cycle: LST: 600 Longitude (if geo): Asc/desc: Ascending URL: winds.jpl.nasa.gov/missions/quikscat/index.cfm |
| # Terra (NASA) | Currently being flown | 18 Dec 99 | 18 Dec 05 | Atmospheric dynamics/water and energy cycles, Atmospheric chemistry, Physical and radiative properties of clouds, airland exchanges of energy, carbon and water, vertical profiles of CO and methane vulcanology | ASTER, CERES, MISR, MODIS, MOPITT | Type: Sun-synchronous Altitude: 705 km Period: 99 mins Inclination: 98.2 deg Repeat cycle: 16 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: terra.nasa.gov/ |
| # ACRIMSAT (NASA) | Currently being flown | 20 Dec 99 | 01 Oct 07 | Will sustain long-term solar luminosity database by providingmeasurements of total solar irradiance and the solar constant | ACRIM III | Type: Sun-synchronous Altitude: 716 km Period: 90 mins Inclination: 98.13 deg Repeat cycle: LST: 1050 Longitude (if geo): Asc/desc: Ascending URL: acrim.jpl.nasa.gov/ |
| # NMP EO-1 (NASA) | Currently being flown | 21 Nov 00 | 30 Sep 05 | Land surface, earth resources | ALI, Hyperion, LAC | Type: Sun-synchronous Altitude: 705 km Period: 99 mins Inclination: 98.2 deg Repeat cycle: 16 days LST: 1030 Longitude (if geo): Asc/desc: Descending URL: eo1.gsfc.nasa.gov/ |
| # TIMED (NASA) | Currently being flown | 07 Dec 01 | 07 Dec 20 | Investigates the influences of the sun and humans on the least explored and understood region of the Earth's atmosphere - the mesosphere and lower thermosphere/ionosphere (MLTI) | SABER | Type: Inclined, non-sunsynchronous Altitude: 625 km Period: Inclination: 74.1 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: URL: www.timed.jhuapl.edu/ |
| # GRACE (NASA) | Currently being flown | 17 Mar 02 | 01 Mar 07 | Extremely high precision gravity measurements for use in construction of gravity field models | GPS (GRACE), HAIRS | Type: Inclined, non-sunsynchronous Altitude: 400 km Period: 94 mins Inclination: 89 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: www.csr.utexas.edu/grace/ |
| # Aqua (NASA) | Currently being flown | 04 May 02 | 04 May 07 | Atmosphericdynamics/water and energy cycles, cloudformation, precipitationand radiativeproperties, air/seafluxes of energy andmoisture, sea ice extentand heat exchange with the atmosphere. Option of 705km or 438km orbit altitude. | AIRS, AMSR-E, AMSU-A, CERES, HSB, MODIS | Type: Sun-synchronous Altitude: 705 km Period: 98.8 mins Inclination: 98.2 deg Repeat cycle: 16 days LST: 1330 Longitude (if geo): Asc/desc: Ascending URL: eos-pm.gsfc.nasa.gov |
| # ICESat (NASA) | Currently being flown | 12 Jan 03 | 30 Jun 05 | Monitors mass balance of polar ice sheets and their contribution to global sea level change. Secondary goals: cloud heights and vertical structure of clouds/aerosols; roughness, reflectivity, vegetation heights, snow-cover. | GLAS | Type: Inclined, non-sunsynchronous Altitude: 600 km Period: 97 mins Inclination: 94 deg Repeat cycle: 183 days LST: Longitude (if geo): Asc/desc: N/A URL: icesat.gsfc.nasa.gov/ |
| # SORCE (NASA) | Currently being flown | 25 Jan 03 | 25 Jan 09 | Continues the precise, long-term measurements of total solar irradiance at UV and VNIR wavelengths. Daily measurements of solar UV. Precise measurements of visible solar irradiance for climate studies. | SIM, SOLSTICE, TIM, XPS | Type: Inclined, non-sunsynchronous Altitude: 600 km Period: Inclination: 40 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: lasp.colorado.edu/sorce/ |
| # Aura (NASA) | Currently being flown | 15 Jul 04 | 15 Jul 10 | Chemistry and dynamics of Earth’s atmosphere from the ground through the mesosphere. | HiRDLS, MLS (EOS-Aura), OMI, TES | Type: Sun-synchronous Altitude: 705 km Period: 98.8 mins Inclination: 98.2 deg Repeat cycle: 16 days LST: 1345 Longitude (if geo): Asc/desc: Ascending URL: aura.gsfc.nasa.gov/ |
| # CloudSat (NASA) | Approved | 15 Apr 05 | 15 Feb 07 | CloudSat will use advanced radar to "slice" through clouds to see their vertical structure, providing a completely new observational capability from space. One of first satellites to study clouds on global basis. Will fly in formation with Aqua and CALIPSO. | CPR (Cloudsat) | Type: Sun-synchronous Altitude: 705 km Period: 98.8 mins Inclination: 98.2 deg Repeat cycle: LST: 1335 Longitude (if geo): Asc/desc: Ascending URL: cloudsat.atmos.colostate.edu/ |
| # NMP EO-3 GIFTS (NASA) | Considered | 01 Oct 06 | 01 Oct 08 | Continuous observation of atmospheric temperature, water vapour content and distribution, and the concentration of certain other atmospheric gases as a function of altitude over time - providing a new way to observe weather and the changing atmosphere. | GIFTS | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -83 Asc/desc: N/A URL: nmp.jpl.nasa.gov/eo3/index.html |
| # OCO (NASA) | Planned | 01 Aug 07 | 01 Aug 09 | High resolution carbon dioxide measurements to characterize sources and sinks on regional scales and quantify their variability over the seasonal cycle | OCO | Type: Sun-synchronous Altitude: 705 km Period: 98.8 mins Inclination: 98.2 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: essp.gsfc.nasa.gov/oco/index.html |
| # Glory (NASA) | Planned | 01 Dec 07 | 01 Dec 10 | Concentration and nature of both natural and anthropogenic aerosols (BC, sulfates, etc.) with accuracy and coverage sufficient for quantification of the aerosol effect on climate, the anthropogenic component of this effect, and the long-term change of this effect caused by natural and anthropogenic factors | APS, TIM | Type: Sun-synchronous Altitude: 824 km Period: 101 mins Inclination: 1030 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: Descending URL: |
| # DSCOVR (NASA) | Approved | 01 Jan 08 | 01 Jan 12 | Continuously observes the sunlit Earth (full disk) – transmitting an image every 15 minutes for distribution by internet. Studies how solar radiation affects climate. Will be positioned at the Langrange point between Earth and sun. | EPIC, NISTAR, Plasma-Mag | Type: TBD Altitude: 1500000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: N/A URL: triana.gsfc.nasa.gov/home/www-pm.larc.nasa.gov/triana.html |
| # GPM Core (NASA) | Planned | 01 Nov 09 | 01 Nov 13 | Study of global precipitation, evaporation, and cycling of water are changing. The mission comprises a primary spacecraft with active and passive microwave instruments, and a number of ‘constellation spacecraft with passive microwave instruments | DPR, GMI | Type: Inclined, non-sunsynchronous Altitude: 400 km Period: Inclination: 65 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: gpm.gsfc.nasa.gov/gpm.gsfc.nasa.gov/ |
| # GPM Constellation (NASA) | Planned | 01 Nov 10 | 01 Nov 14 | Study of global precipitation, evaporation, and cycling of water are changing. The mission comprises a primary spacecraft with active and passive microwave instruments, and a number of ‘constellation spacecraft with passive microwave instruments. | GMI | Type: Inclined, non-sunsynchronous Altitude: 600 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: gpm.gsfc.nasa.gov/ |
| # HYDROS (NASA) | Planned | 01 Dec 10 | 01 Dec 14 | Measure soil moisture content and freeze-thaw state | HYDROS | Type: Sun-synchronous Altitude: 670 km Period: Inclination: 98 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: hydros.gsfc.nasa.gov/ |
| # LAGEOS-2 (NASA / ASI) | Currently being flown | 22 Oct 92 | 22 Oct 32 | Geodesy, crustal motion and gravity field measurements by laser ranging | LRA (LAGEOS) | Type: Inclined, non-sunsynchronous Altitude: 5900 km Period: 223 mins Inclination: 52 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: N/A URL: ilrs.gsfc.nasa.gov/satellite_missions/list_of_satellites/lagoes.htmlNASA\NASA_Agency_Missions1x2.doc |
| # LAGEOS-3 (NASA / ASI) | Planned | 01 Jan 06 | 01 Jan 46 | Geodesy, crustal motion and gravity field measurements by laser ranging. Launch TBD - dates given are for illustration only. | LRA (LAGEOS) | Type: Inclined, non-sunsynchronous Altitude: 5900 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: www.laeff.esa.es/eng/laeff/activity/lageos3.html |
| # Topex-Poseidon (NASA / CNES) | Currently being flown | 10 Aug 92 | 01 Oct 05 | Physical oceanography, geodesy/gravity | DORIS, GPSDR, LRA, POSEIDON-1 (SSALT-1), TMR, TOPEX | Type: Inclined, non-sunsynchronous Altitude: 1336 km Period: 112.4 mins Inclination: 66 deg Repeat cycle: 10 days LST: Longitude (if geo): Asc/desc: N/A URL: topex-www.jpl.nasa.gov/mission/tp-launch.html |
| # Jason (NASA / CNES) | Currently being flown | 07 Dec 01 | 01 Oct 07 | Physical oceanography, geodesy/gravity, climate monitoring, marine meteorology | DORIS-NG, JMR, LRA, POSEIDON-2 (SSALT-2), TRSR | Type: Inclined, non-sunsynchronous Altitude: 1336 km Period: 112.4 mins Inclination: 66 deg Repeat cycle: 10 days LST: Longitude (if geo): Asc/desc: N/A URL: topex-www.jpl.nasa.gov/mission/jason-1.html |
| # CALIPSO (NASA / CNES) | Approved | 15 Apr 05 | 15 Apr 08 | Measurements of aerosol & cloud properties for climate predictions, using a 3 channel lidar and passive instruments in formation with Aqua and CloudSat for coincident observations of radiative fluxes and atmospheric state. | CALIOP, IIR, WFC | Type: Sun-synchronous Altitude: 705 km Period: 98.8 mins Inclination: 98.2 deg Repeat cycle: LST: 1330 Longitude (if geo): Asc/desc: Ascending URL: www-calipso.larc.nasa.gov/ |
| # Jason-2 (also known as OSTM) (NASA / CNES) | Planned | 07 Dec 06 | 07 Dec 11 | Physical oceanography, geodesy/gravity, climate monitoring, marine meteorology | DORIS-NG, JMR, LRA, POSEIDON-3, TRSR | Type: Inclined, non-sunsynchronous Altitude: 1336 km Period: 112.4 mins Inclination: 66 deg Repeat cycle: 10 days LST: Longitude (if geo): Asc/desc: N/A URL: topex-www.jpl.nasa.gov/mission/mission.html |
| # NOAA-12 (NOAA) | Currently being flown | 14 May 91 | 31 Dec 05 | Meteorology, agriculture and forestry, environmental monitoring, climatology, physical oceanography, Volcanic eruption monitoring, ice and snow cover, total ozone studies, space environment, solar flux analysis | ARGOS, AVHRR/2, HIRS/2, MSU, NOAA Comms, SEM (POES), SSU | Type: Sun-synchronous Altitude: 850 km Period: 101.3 mins Inclination: 98.5 deg Repeat cycle: LST: 449 Longitude (if geo): Asc/desc: Descending URL: www.oso.noaa.gov/poes/ |
| # NOAA-14 (NOAA) | Currently being flown | 30 Dec 94 | 31 Dec 05 | Meteorology, agriculture and forestry, environmental monitoring, climatology, physical oceanography, Volcanic eruption monitoring, ice and snow cover, total ozone studies, space environment, solar flux analysis | ARGOS, AVHRR/2, HIRS/2, MSU, NOAA Comms, S&R (NOAA), SBUV/2, SEM (POES), SSU | Type: Sun-synchronous Altitude: 850 km Period: 102.1 mins Inclination: 99.1 deg Repeat cycle: LST: 1752 Longitude (if geo): Asc/desc: Ascending URL: www.oso.noaa.gov/poes/ |
| # GOES-9 (NOAA) | Currently being flown | 23 May 95 | 15 Jan 05 | Meteorology (primary mission), search and rescue, space environment monitoring, data collection, platform, data gathering, WEFAX | DCS (NOAA), GOES Comms, Imager, S&R (GOES), SEM (GOES), Sounder, WEFAX | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 255 Asc/desc: N/A URL: www.oso.noaa.gov/goes/ |
| # DMSP F-13 (NOAA) | Currently being flown | 01 Mar 97 | 30 Sep 05 | The long-term meteorological programme of the US Department of Defense (DoD) - with the objective to collect and disseminate worldwide atmospheric, oceanographic, solar-geophyscial, and cloud cover data on a daily basis. (Primary operational satellite) | OLS, SSB/X-2, SSIES-2, SSJ/4, SSM, SSM/I, SSM/T-1, SSM/T-2, SSZ | Type: Sun-synchronous Altitude: 833 km Period: 101 mins Inclination: 98.7 deg Repeat cycle: LST: 1812 Longitude (if geo): Asc/desc: Ascending URL: dmsp.ngdc.noaa.gov/dmsp.html |
| # GOES-10 (NOAA) | Currently being flown | 25 Apr 97 | 15 Jan 06 | Meteorology (primary mission), search and rescue, space environment monitoring, data collection platform, data gathering, WEFAX | DCS (NOAA), GOES Comms, Imager, S&R (GOES), SEM (GOES), Sounder, WEFAX | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 135 Asc/desc: N/A URL: www.oso.noaa.gov/goes/ |
| # NOAA-15 (NOAA) | Currently being flown | 01 May 98 | 31 Dec 06 | Meteorology, agriculture and forestry, environmental monitoring, climatology, physical oceanography, Volcanic eruption monitoring, ice and snow cover, space environment, solar flux analysis, search and rescue | AMSU-A, AMSU-B, ARGOS, AVHRR/3, HIRS/3, NOAA Comms, S&R (NOAA), SBUV/2, SEM (POES) | Type: Sun-synchronous Altitude: 813 km Period: 101.4 mins Inclination: 98.6 deg Repeat cycle: LST: 708 Longitude (if geo): Asc/desc: Descending URL: www.oso.noaa.gov/poes/ |
| # DMSP F-15 (NOAA) | Currently being flown | 12 Dec 99 | 30 Sep 06 | The long-term meteorological programme of the US Department of Defense (DoD) - with the objective to collect and disseminate worldwide cloud cover data on a daily basis. (Primary operational satellite) | OLS, SSB/X-2, SSIES-2, SSJ/4, SSM, SSM/I, SSM/T-1, SSM/T-2, SSZ | Type: Sun-synchronous Altitude: 833 km Period: 101 mins Inclination: 98.9 deg Repeat cycle: LST: 2029 Longitude (if geo): Asc/desc: Ascending URL: dmsp.ngdc.noaa.gov/dmsp.html |
| # GOES-11 (NOAA) | Currently being flown | 03 May 00 | 15 Jul 11 | Meteorology (primary mission), search and rescue, space environment monitoring, data collection platform, data gathering, WEFAX | DCS (NOAA), GOES Comms, Imager, S&R (GOES), SEM (GOES), Sounder, WEFAX | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 103 Asc/desc: N/A URL: www.oso.noaa.gov/goes/ |
| # NOAA-16 (NOAA) | Currently being flown | 21 Sep 00 | 31 Dec 06 | Meteorology, agriculture and forestry, environmental monitoring, climatology, physical oceanography, Volcanic eruption monitoring, ice and snow cover, total ozone studies, space environment, solar flux analysis, search and rescue | AMSU-A, AMSU-B, ARGOS, AVHRR/3, HIRS/3, NOAA Comms, S&R (NOAA), SBUV/2, SEM (POES) | Type: Sun-synchronous Altitude: 870 km Period: 102 mins Inclination: 98.8 deg Repeat cycle: LST: 1354 Longitude (if geo): Asc/desc: Ascending URL: www.oso.noaa.gov/poes/ |
| # GOES-12 (NOAA) | Currently being flown | 23 Jul 01 | 15 Jan 11 | Meteorology (primary mission), search and rescue, space environment monitoring, data collection platform, data gathering, WEFAX | DCS (NOAA), GOES Comms, Imager, S&R (GOES), SEM (GOES), Sounder, SXI, WEFAX | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 75 Asc/desc: N/A URL: www.oso.noaa.gov/goes/ |
| # NOAA-17 (NOAA) | Currently being flown | 24 Jun 02 | 01 Mar 06 | Meteorology, agriculture and forestry, environmental monitoring, climatology, physical oceanography, Volcanic eruption monitoring, ice and snow cover, total ozone studies, space environment, solar flux analysis, search and rescue | AMSU-A, AMSU-B, ARGOS, AVHRR/3, HIRS/3, NOAA Comms, S&R (NOAA), SBUV/2, SEM (POES) | Type: Sun-synchronous Altitude: 833 km Period: 101.4 mins Inclination: 98.75 deg Repeat cycle: LST: 1000 Longitude (if geo): Asc/desc: Descending URL: www.oso.noaa.gov/poes/ |
| # DMSP F-16 (NOAA) | Approved | 15 Oct 03 | 15 Oct 07 | The long-term meteorological programme of the US Department of Defense (DoD) - with the objective to collect and disseminate worldwide cloud cover data on a daily basis. | OLS, SSB/X-2, SSIES-2, SSIES-3, SSJ/4, SSJ/5, SSM, SSM/I, SSM/T-1, SSM/T-2, SSMIS, SSULI, SSUSI, SSZ | Type: Sun-synchronous Altitude: 833 km Period: 101 mins Inclination: 98.9 deg Repeat cycle: LST: 2132 Longitude (if geo): Asc/desc: Ascending URL: dmsp.ngdc.noaa.gov/dmsp.html |
| # NOAA-N (NOAA) | Approved | 11 Feb 05 | 30 Nov 08 | Meteorology, agriculture and forestry, environmental monitoring, climatology, physical oceanography, Volcanic eruption monitoring, ice and snow cover, total ozone studies, space environment, solar flux analysis, search and rescue | AMSU-A, ARGOS, AVHRR/3, HIRS/4, MHS, NOAA Comms, S&R (NOAA), SBUV/2, SEM (POES) | Type: Sun-synchronous Altitude: 870 km Period: 102.1 mins Inclination: 98.75 deg Repeat cycle: LST: 1400 Longitude (if geo): Asc/desc: Ascending URL: www.oso.noaa.gov/poes/ |
| # GOES-N (NOAA) | Approved | 12 Feb 05 | 12 Feb 17 | Meteorology (primary mission), search and rescue, space environment monitoring, data collection platform, data gathering, WEFAX | DCS (NOAA), GOES Comms, Imager, S&R (GOES), SEM (GOES), Sounder, SXI, WEFAX | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 75 Asc/desc: N/A URL: www.oso.noaa.gov/goes/ |
| # DMSP F-17 (NOAA) | Approved | 15 Apr 05 | 01 Jun 08 | The long-term meteorological programme of the US Department of Defense (DoD) - with the objective to collect and disseminate worldwide cloud cover data on a daily basis. | OLS, SSIES-3, SSJ/5, SSM, SSM/T-1, SSMIS, SSULI, SSUSI | Type: Sun-synchronous Altitude: 850 km Period: 101 mins Inclination: 98.7 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: Ascending URL: dmsp.ngdc.noaa.gov/dmsp.html |
| # NPP (NOAA) | Approved | 31 Oct 06 | 26 Oct 11 | Meteorological, climatic, terrestrial, and oceanographic applications; global and regional environmental monitoring. (Joint mission with NASA) | ATMS, CrIS, OMPS, VIIRS | Type: Sun-synchronous Altitude: 833 km Period: 101 mins Inclination: 1030 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: Descending URL: jointmission.gsfc.nasa.gov/ |
| # GOES-O (NOAA) | Approved | 07 Apr 07 | 07 Apr 19 | Meteorology (primary mission), search and rescue, space environment monitoring, data collection platform, data gathering, WEFAX | DCS (NOAA), GOES Comms, Imager, S&R (GOES), SEM (GOES), Sounder, WEFAX | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 75 Asc/desc: TBD URL: www.oso.noaa.gov/goes/ |
| # DMSP F-18 (NOAA) | Approved | 15 Oct 07 | 15 Oct 10 | The long-term meteorological programme of the US Department of Defense (DoD) - with the objective to collect and disseminate worldwide cloud cover data on a daily basis. | OLS, SSIES-3, SSJ/5, SSM, SSM/T-1, SSMIS, SSULI, SSUSI | Type: Sun-synchronous Altitude: 850 km Period: 101 mins Inclination: 98.7 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: Ascending URL: dmsp.ngdc.noaa.gov/dmsp.html |
| # GOES-P (NOAA) | Approved | 15 Oct 08 | 15 Oct 20 | Meteorology (primary mission), search and rescue, space environment monitoring, data collection platform, data gathering, WEFAX | DCS (NOAA), GOES Comms, Imager, S&R (GOES), SEM (GOES), Sounder, SXI, WEFAX | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): 135 Asc/desc: TBD URL: www.oso.noaa.gov/goes/ |
| # NOAA-N' (NOAA) | Approved | 30 Nov 08 | 01 Dec 11 | Meteorology, agriculture and forestry, environmental monitoring, climatology, physical oceanography, Volcanic eruption monitoring, ice and snow cover, total ozone studies, space environment, solar flux analysis, search and rescue | AMSU-A, ARGOS, AVHRR/3, HIRS/4, MHS, NOAA Comms, S&R (NOAA), SBUV/2, SEM (POES) | Type: Sun-synchronous Altitude: 870 km Period: 102.1 mins Inclination: 98.75 deg Repeat cycle: LST: 1400 Longitude (if geo): Asc/desc: Ascending URL: www.oso.noaa.gov/poes/ |
| # DMSP F-19 (NOAA) | Approved | 15 Apr 09 | 15 Apr 12 | The long-term meteorological programme of the US Department of Defense (DoD) - with the objective to collect and disseminate worldwide cloud cover data on a daily basis. | OLS, SSIES-3, SSJ/5, SSM, SSM/T-1, SSMIS, SSULI, SSUSI | Type: Sun-synchronous Altitude: 833 km Period: 101 mins Inclination: 98.7 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: Ascending URL: dmsp.ngdc.noaa.gov/dmsp.html |
| # NPOESS-1 (NOAA) | Approved | 30 Nov 09 | 01 Nov 15 | Meteorological, climatic, terrestrial, oceanographic, and solar-geophysical applications; global and regional environmental monitoring, search and rescue, data collection. | A-DCS, APS, CMIS, SARSAT, VIIRS | Type: Sun-synchronous Altitude: 833 km Period: 101 mins Inclination: 98.75 deg Repeat cycle: 2130 days LST: Longitude (if geo): Asc/desc: Ascending URL: www.npoess.noaa.gov/ |
| # NPOESS-2 (NOAA) | Approved | 30 Jun 11 | 01 Jun 17 | Meteorological, climatic, terrestrial, oceanographic, and solar-geophysical applications; global and regional environmental monitoring, search and rescue, data collection. | A-DCS, ATMS, CERES, CMIS, CrIS, OMPS, S&R (NOAA), SARSAT, SESS, VIIRS | Type: Sun-synchronous Altitude: 833 km Period: 101 mins Inclination: 98.75 deg Repeat cycle: 1330 days LST: Longitude (if geo): Asc/desc: Ascending URL: www.npoess.noaa.gov/ |
| # DMSP F-20 (NOAA) | Approved | 15 Oct 11 | 15 Oct 14 | The long-term meteorological programme of the US Department of Defense (DoD) - with the objective to collect and disseminate worldwide cloud cover data on a daily basis. | OLS, SSIES-3, SSJ/5, SSM, SSM/T-1, SSMIS, SSULI, SSUSI | Type: Sun-synchronous Altitude: 850 km Period: 101 mins Inclination: 98.7 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: Ascending URL: dmsp.ngdc.noaa.gov/dmsp.html |
| # GOES-R (NOAA) | Approved | 31 Oct 12 | 31 Oct 20 | Meteorology (primary mission), search and rescue, space environment monitoring, data collection platform, data gathering, WEFAX | ABI, EHIS, EUVS, Geomicrowave sounder, GLM, HES, Magnetometer (NOAA), MPS, SGPS, SXI, SXS | Type: Geostationary Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): 135 Asc/desc: URL: www.osd.noaa.gov/goes_R/index.htm |
| # NPOESS-3 (NOAA) | Approved | 04 Jun 13 | 09 Jun 18 | Meteorological, climatic, terrestrial, oceanographic, and solar-geophysical applications; global and regional environmental monitoring, search and rescue, data collection. | A-DCS, ALT, CMIS, ERBS, S&R (NOAA), SARSAT, SESS, TSIS, VIIRS | Type: Sun-synchronous Altitude: 833 km Period: 101 mins Inclination: 98.75 deg Repeat cycle: 1730 days LST: Longitude (if geo): Asc/desc: Ascending URL: www.npoess.noaa.gov/ |
| # NPOESS-4 (NOAA) | Approved | 01 Nov 15 | 06 Nov 20 | Meteorological, climatic, terrestrial, oceanographic, and solar-geophysical applications; global and regional environmental monitoring, search and rescue, data collection. | A-DCS, APS, CMIS, SARSAT, SESS, VIIRS | Type: Sun-synchronous Altitude: 833 km Period: 101 mins Inclination: 98.75 deg Repeat cycle: 2130 days LST: Longitude (if geo): Asc/desc: Ascending URL: www.npoess.noaa.gov/ |
| # NPOESS-5 (NOAA) | Approved | 01 Jan 18 | 01 Jun 23 | Meteorological, climatic, terrestrial, oceanographic, and solar-geophysical applications; global and regional environmental monitoring, search and rescue, data collection. | A-DCS, ATMS, CERES, CMIS, CrIS, OMPS, S&R (NOAA), SARSAT, SESS, VIIRS | Type: Sun-synchronous Altitude: 833 km Period: 101 mins Inclination: 98.75 deg Repeat cycle: 1330 days LST: Longitude (if geo): Asc/desc: Ascending URL: www.npoess.noaa.gov/ |
| # NPOESS-6 (NOAA) | Approved | 09 May 19 | 01 May 24 | Meteorological, climatic, terrestrial, oceanographic, and solar-geophysical applications; global and regional environmental monitoring, search and rescue, data collection. | A-DCS, ALT, CMIS, ERBS, S&R (NOAA), SARSAT, SESS, TSIS, VIIRS | Type: Sun-synchronous Altitude: 833 km Period: 101 mins Inclination: 98.75 deg Repeat cycle: 1730 days LST: Longitude (if geo): Asc/desc: Ascending URL: www.npoess.noaa.gov/ |
| # FY-2C (NRSCC) | Currently being flown | 19 Oct 04 | 19 Oct 07 | Meteorology and environmental monitoring Data collection and redistribution | IVISSR (FY-2) | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -105 Asc/desc: TBD URL: |
| # FY-3A (NRSCC) | Planned | 01 Jan 06 | 01 Jan 08 | Meteorology and environmental monitoring Data collection and redistribution | IRAS, MERSI, MWRI, MWTS, OP, TOM, VIRR | Type: Sun-synchronous Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: |
| # FY-2D (NRSCC) | Planned | 31 Dec 06 | 31 Dec 09 | Meteorology and environmental monitoring Data collection and redistribution | IVISSR (FY-2) | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -105 Asc/desc: TBD URL: |
| # FY-3B (NRSCC) | Planned | 31 Dec 06 | 31 Dec 08 | Meteorology and environmental monitoring Data collection and redistribution | IRAS, MERSI, MWRI, MWTS, OP, TOM, VIRR | Type: Sun-synchronous Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: |
| # FY-3C (NRSCC) | Planned | 31 Dec 08 | 31 Dec 10 | Meteorology and environmental monitoring Data collection and redistribution | IMWTS, IRAS, MERSI, MIRAS, MWHS, MWRI, OP, TOM, VIRR | Type: Sun-synchronous Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: |
| # FY-2E (NRSCC) | Planned | 31 Dec 09 | 31 Dec 12 | Meteorology and environmental monitoring Data collection and redistribution | IVISSR (FY-2) | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -105 Asc/desc: TBD URL: |
| # FY-3D (NRSCC) | Planned | 31 Dec 10 | 31 Dec 12 | Meteorology and environmental monitoring Data collection and redistribution | IMWTS, IRAS, MERSI, MIRAS, MWHS, MWRI, OP, TOM, VIRR | Type: Sun-synchronous Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: |
| # FY-3E (NRSCC) | Planned | 31 Dec 12 | 31 Dec 14 | Meteorology and environmental monitoring Data collection and redistribution | IMWTS, IRAS, MERSI, MIRAS, MWHS, MWRI, OP, TOM, VIRR | Type: Sun-synchronous Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: |
| # FY-3F (NRSCC) | Planned | 31 Dec 14 | 31 Dec 16 | Meteorology and environmental monitoring Data collection and redistribution | IMWTS, IRAS, MERSI, MIRAS, MWHS, MWRI, OP, TOM, VIRR | Type: Sun-synchronous Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: |
| # FY-3G (NRSCC) | Planned | 31 Dec 16 | 31 Dec 18 | Meteorology and environmental monitoring Data collection and redistribution | IMWTS, IRAS, MERSI, MIRAS, MWHS, MWRI, OP, TOM, VIRR | Type: Sun-synchronous Altitude: Period: Inclination: Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: |
| # SICH-2 (NSAU) | Planned | 31 Dec 05 | 31 Dec 10 | Land Observation | MBEI, MIREI | Type: Sun-synchronous Altitude: 668 km Period: 98 mins Inclination: 98 deg Repeat cycle: 4 days LST: 1050 Longitude (if geo): Asc/desc: TBD URL: |
| # SICH-1M (NSAU / ROSKOSMOS) | Approved | 31 Dec 04 | 31 Dec 07 | Physical Oceanography, Hydrometeorology, Land Observation | MSU-EU, MSU-M, MTVZA-OK, RLSBO, RM-08 | Type: Inclined, non-sunsynchronous Altitude: 650 km Period: 98 mins Inclination: 82.5 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: |
| # METEOR-3M N1 (ROSHYDROMET / ROSKOSMOS) | Currently being flown | 10 Dec 01 | 31 Dec 05 | Hydrometeorology, climatology, land surface, physical oceanography, heliogeophysics and space environment, sounding of the atmosphere, agriculture. (Expected operational during 2002). | KGI-4C, Klimat, MIVZA, MR-2000M1, MSTE-5E, MSU-E, MSU-SM, MTVZA, SAGE III, SFM-2 | Type: Sun-synchronous Altitude: 1018 km Period: 105.3 mins Inclination: 99.6 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: Ascending URL: sputnik1.infospace.ru/ |
| # Meteor-M No1 (ROSHYDROMET / ROSKOSMOS) | Planned | 01 Jan 05 | 01 Jan 12 | Meteorology, hydrology, climate and environmental monitoring | GALS-M, GGAK-M, IKFS-2, IKOR-M, KMSS, MSGI-MKA, MSS-BIO, MSU-MR, MTVZA, Radiomet, RIMS-M, SKL-M | Type: Sun-synchronous Altitude: 835 km Period: Inclination: 98.8 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: URL: www.federalspace.ru |
| # METEOR-3M N2 (ROSHYDROMET / ROSKOSMOS) | Approved | 01 Dec 05 | 31 Dec 09 | Hydrometeorology, climatology, land surface, physical oceanography, heliogeophysics and space environment, data collection, sounding of the atmosphere, agriculture | DCS (ROSHYDROMET), IKFS-2, KMSS, MSGI-MKA, MSU-MR, MTVZA, SAR (ROSHYDROMET) | Type: Sun-synchronous Altitude: 1024 km Period: 105.3 mins Inclination: 99.6 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: Ascending URL: sputnik1.infospace.ru/ |
| # Elektro-L (ROSHYDROMET / ROSKOSMOS) | Approved | 01 Dec 06 | 31 Dec 12 | Hydrometeorology, climatology, disaster management, space environment, ice and snow, land surface, space environment, data collection and communication. | DCS (ROSHYDROMET), GGAK-E, MSU-GS | Type: Geostationary Altitude: 36000 km Period: Inclination: Repeat cycle: LST: Longitude (if geo): -76 Asc/desc: N/A URL: sputnik1.infospace.ru/ |
| # Meteor-M No2 (ROSHYDROMET / ROSKOSMOS) | Planned | 01 Jan 07 | 01 Jan 14 | Meteorology, hydrology, climate and environmental monitoring | GALS-M, GGAK-M, IKFS-2, IKOR-M, KMSS, MSGI-MKA, MSS-BIO, MSU-MR, MTVZA, Radiomet, RIMS-M, SKL-M | Type: Sun-synchronous Altitude: 835 km Period: Inclination: 98.8 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: URL: www.federalspace.ru |
| # Vulkan-Kompas-2 (ROSKOSMOS) | Planned | 01 Jan 05 | 31 Dec 06 | Monitoring of Earth's seismic activity for earthquake and volcanic eruption forecasting | DRF, GID-12T, NVK, RBE, RCHA | Type: Inclined, non-sunsynchronous Altitude: 500 km Period: Inclination: 79 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: URL: www.federalspace.ru |
| # Monitor-E (ROSKOSMOS) | Planned | 01 Apr 05 | 31 Dec 10 | Agriculture and forestry, hydrology, environmental monitoring, hydrometeorology, ice and snow, land surface, meteorology | PSA, RDSA | Type: Sun-synchronous Altitude: 540 km Period: Inclination: 97.5 deg Repeat cycle: LST: 540 Longitude (if geo): Asc/desc: TBD URL: www.federalspace.ru |
| # Resurs DK (ROSKOSMOS) | Planned | 01 Jun 05 | 31 Dec 08 | Agriculture and forestry, hydrology, environmental monitoring, hydrometeorology, ice and snow, land surface, meteorology | Arina, Geoton-L1, Pamela | Type: Inclined, non-sunsynchronous Altitude: 480 km Period: Inclination: 70.4 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: TBD URL: sputnik1.infospace.ru/ |
| # Baumanets (ROSKOSMOS) | Planned | 05 Dec 05 | 31 Dec 06 | Experimental satellite for data transmission, remote sensing, and spacecraft management. | OEA | Type: Sun-synchronous Altitude: 700 km Period: Inclination: 98 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: URL: www.federalspace.ru |
| # BelKA (ROSKOSMOS) | Planned | 01 Jan 06 | 31 Dec 11 | Regular imaging of terrestrial sites in visible and near IR ranges with high spatial resolution | MSS (Roskosmos), PSS | Type: Sun-synchronous Altitude: 505 km Period: Inclination: 97.4 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: URL: www.federalspace.ru |
| # Kanopus-Vulkan (ROSKOSMOS) | Planned | 01 Jan 06 | 31 Dec 11 | Hydrology, hydrometeorology, monitoring man-made and natural accidents, research into short-term forecasting of earthquakes | ECHO-V, GID-12T, MTVZA-OK, NVK, RBE, RCHA | Type: Sun-synchronous Altitude: 700 km Period: Inclination: 97 deg Repeat cycle: LST: Longitude (if geo): Asc/desc: URL: www.federalspace.ru |
| # Odin (SNSB) | Currently being flown | 20 Feb 01 | 30 Mar 05 | Atmospheric research, stratospheric ozone chemistry, mesospheric ozone science, summer mesospheric science | OSIRIS, SMR | Type: Sun-synchronous Altitude: 590 km Period: 97.6 mins Inclination: 97.8 deg Repeat cycle: LST: 1800 Longitude (if geo): Asc/desc: Ascending URL: www.ssc.se |
| # Landsat-5 (USGS) | Currently being flown | 01 Mar 84 | 31 Dec 09 | Earth resources, land surface, environmental monitoring, agriculture and forestry, disaster monitoring and assessment, ice and snow cover | Landsat Comms, MSS, TM | Type: Sun-synchronous Altitude: 705 km Period: 98.9 mins Inclination: 98.2 deg Repeat cycle: 16 days LST: 945 Longitude (if geo): Asc/desc: Descending URL: landsat7.usgs.gov/ |
| # Landsat-7 (USGS) | Currently being flown | 15 Apr 99 | 31 Dec 10 | Earth resources, land surface, environmental monitoring, agriculture and forestry, disaster monitoring and assessment, ice and snow cover | ETM+, Landsat Comms | Type: Sun-synchronous Altitude: 705 km Period: 98.8 mins Inclination: 98.2 deg Repeat cycle: 16 days LST: 1000 Longitude (if geo): Asc/desc: Descending URL: landsat7.usgs.gov/ |